| Employee Name or Team: |  |
| --- | --- |
| Job Title: |  |
| Service: |  |
| Assessing Manager’s Name: |  |
| Assessing Manager’s Job Title: |  |
| Date Assessment Completed: |  |

| Risk Definitions | Assessment |
| --- | --- |
| L | Low : minor injuries requiring first aid – for example, grazes or minor cuts |
| M | Medium: an injury requiring further medical assistance – for example cuts needing stiches or broken bones |
| H | High: major injuries, such as paralysis, or death |
| N/A | Not applicable to current job |
| Residual Risk | Residual risk is the level of risk that remains after suitable control measures are introduced |

Assess the risk of each activity between Low, Medium or High.  Following introduction of risk control measures reassess for residual risk.

| Activity | Person at risk | Significant Hazards | Risk L M H |  |  | Risk Control Measures | Residual Risk L M H |  |  | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Erecting a tower scaffold | Operative | Falls from height, electric shocks |  |  |  | • Tower to be erected as per the “through the trap” (3T) or advanced guard rail system as recommended by the HSE • Tower scaffolds to be erected by trained personnel only, i.e. those who have been on a course approved by PASMA • Operatives not to climb on the outside of the tower at any time • Towers to only be erected on solid, secure standing • Towers not to be erected underneath power lines Guard rails, edge protection and stabilisers to be used where necessary |  |  |  |  |
| Working at height from a tower scaffold | As above | Falls from height |  |  |  | • Those using the tower to stay within the confines of the tower at all times • The tower not to be overloaded with equipment • No parts of the tower to be removed or adjusted whilst in use • Equipment not to be hoisted up the side of the tower |  |  |  |  |
| Moving a tower scaffold | As above | Toppling, crushing those below |  |  |  | • Path to be cleared of all debris before tower moved • Ground to be checked for stability • Scaffolds to be checked on a regular basis for stability etc. by trained personnel |  |  |  |  |
| Dismantling a tower scaffold | As above | Falls from height, toppling, crushing those below |  |  |  | • Tower scaffolds to be dismantled, moved and re-erected by trained personnel only • Tower to be dismantled as per the “through the trap” (3T) method or advanced guard rail system as recommended by the HSE |  |  |  |  |
| Any other comments |  |  |  |  |  |  |  |  |  |  |

| #### Sign and file for reference |  |  |  |
| --- | --- | --- | --- |
| #### Employee’s Signature: |  | #### Date: |  |
| #### Line Manager’s Signature: |  | #### Date: |  |

Date for revision: Annually or if any significant changes occur - Ensure review date recorded	    	                                             December 2018